LA3 MAF Conversion BackFlow Fix…..    			1/21/06

Version 3 						Updated 11/18/06

Update Notes added 11/18/06:

Over time I started lowering the clip values in order improve the closed throttle idle even more after boost. I found the startup enrichments were being affected by the Maf  clip and the motor would stall when first started warm or cold.  So I added a very simple delay to the clip, only 5 new lines of EEC code. The Maf Clip will not be active for up to  90 secs after the motor is started. This allows enough time for normal A/F controls to take over and prevents any stalling.  Also the delay can be adjust via the new .RDT parameter provided below.   Still, if you adjust the MAF clip to low…the motor simply will not idle…so be care in adjusting the MAF clip.

Version 2 						Updated 9/8/06

Update Notes added 9/8/06:

The .RDT scaling factor for the “backflow maf Clip vs Rpm” must be adjusted to match the Mass Air Multipler at P_1220 PCMX Address  or 3220 real EEC address. Stock the Multipler is “1”  ( 8000 hex ). Depending on how much  you have setup your bin to flow, it can be set anywhere from “1”  to flow a max of 32 #s  or as high as “2” (ffff hex ) to flow a max of 64#s/min. The scaling of the Y axis should be set to 1024 for max flow of 64#s, which is what I provided in the example on page  5 of this doc. If you run the stock max flow number of 1, then the scaling show be 2048.

If your Mass Air Multipler is set between 1 & 2, then you need to do the math…..

Y scaling = 65535 /  (   “Your Multipler” / 2    *  64   )

The sample numbers provided in the   “backflow maf Clip vs Rpm” are good starting points, they worked for me. However recently I have reduced the #s / min even a little more.

By John K…..

Goal:  After converting to a MAF sensor, some people (myself included) have experienced an idle stall immediately after a HIGH boost run.  This would occur once the throttle is shut and the motor is allowed to idle with the clutch disengaged. The reason behind this is that air is actually backing out of the turbo ( i.e. Backflow ) and being measured by the MAF & EEC as actual flow into the motor. Keep in mind the VAM will indicate zero flow if air is flow backwards.  This causes a rich A/F mixture. The Rich A/F causes stalling depending on how bad the back flow is.  I was able to improve the issue by adjusting the Idle Mass flow Clip, MAX Decel PW and variety of other tweaks, but never totally eliminated it.

The back flow is affected by a number of factors. This includes:

- Size of MAF

- Distance between the turbo and MAF

- Use of a BOV or BPV may  improve or worsen the behavior

- Size of turbo

- How much boost

- IC size

The Fix:

This drove me to add a closed throttle MAX MAF clip to the LA3 code.

Below are the .EEC code and .rdt additions.   The Code uses P_2478 (same as the stock code) to determine what is a open vs.  a closed throttle.   The addition to the .RDT is the Function that determines what level to CLIP the MAF to based on the RPM (This may need tuning for your application). The code is resident at 8250 real hex address (6250 PCMX ), so you will need to check if you have used that address already ( most likely not ) and the Function is added at 8290.  Add the .eec via the tuner emulator and then just use an editor to modify the .rdt. Then adjust the new the function MAF clip function to values I provided.

Results:

I have been running this about a week. The rich condition after boost is gone. The A/F meter returns to switching almost as soon as the fuel is turned back on from the Decel Fuel Shutoff function. The idle is almost identical as a VAM after high boost. Prior to this mod I had an occasional closed throttle backfire out the exhaust which is also gone.

If you give this a try, post feed back on this site on the results….I am thinking of adding a cold motor bypass to this.  ( SEE VERSION 3 NOTES ADD ON PAGE 1 )

.EEC File of the code that needs to be added. FOR MAFS THAT STILL USE THE AIR MASS TRANSFER

all

0x4374  0xEF

0x4375  0xD9

0x4376  0x3E

0x8250  0xA0

0x8251  0x3E

0x8252  0x74

0x8253  0x99

0x8254  0x78

0x8255  0x49

0x8256  0xD3

0x8257  0x1F

0x8258  0xA0

0x8259  0x78

0x825A  0x34

0x825B  0x67

0x825C  0x01

0x825D  0x78

0x825E  0x24

0x825F  0x34

0x8260  0x88

0x8261  0x34

0x8262  0x90

0x8263  0xDB

0x8264  0x12

0x8265  0xA1

0x8266  0x90

0x8267  0x82

0x8268  0x32

0x8269  0xA0

0x826A  0x70

0x826B  0x34

0x826C  0xEF

0x826D  0x8C

0x826E  0xD6

0x826F  0x88

0x8270  0x38

0x8271  0x74

0x8272  0xD3

0x8273  0x03

0x8274  0xA0

0x8275  0x38

0x8276  0x74

0x8277  0xF0

end

.EEC File of the code that needs to be added. FOR MAFS THAT DOES NOT USE THE AIR MASS TRANSFER – STRAIGHT MAF TRANSFER TO #/MIN

All

0x433d  0x36

0x4374  0xEF

0x4375  0xD9

0x4376  0x3E

0x8250  0xFF

0x8251  0xFF

0x8252  0xFF

0x8253  0x99

0x8254  0x78

0x8255  0x49

0x8256  0xD3

0x8257  0x1F

0x8258  0xA0

0x8259  0x78

0x825A  0x34

0x825B  0x67

0x825C  0x01

0x825D  0x78

0x825E  0x24

0x825F  0x34

0x8260  0x88

0x8261  0x34

0x8262  0x90

0x8263  0xDB

0x8264  0x12

0x8265  0xA1

0x8266  0x90

0x8267  0x82

0x8268  0x32

0x8269  0xA0

0x826A  0x70

0x826B  0x34

0x826C  0xEF

0x826D  0x8C

0x826E  0xD6

0x826F  0x88

0x8270  0x38

0x8271  0x74

0x8272  0xD3

0x8273  0x03

0x8274  0xA0

0x8275  0x38

0x8276  0x74

0x8277  0xF0

end

ADDED BACKFLOW CLIP FUNCTION TO THE PCMX  .RDT FILE

BEGIN FUNC "F_6290 Backflow MAF clip vs RPM"

ID "F_6290"

SIZE 7

XLABEL "Engine speed"

XADDRESS 0x6290

XTYPE 0x22

XSTEP -4

XUNITS "RPM"

XFORMAT "%2.0f"

XMATH 1

XSCALE 4.000000

XOFFSET 0.000000

XMIN -500.000000

XMAX 6000.000000

XADJUST 1

XALTADJUST 1

YLABEL "Max MAF"

YADDRESS 0x6292

YTYPE 0x22

YSTEP -4

YUNITS "#/min"

YFORMAT "%1.1f"

YMATH 1

YSCALE 1024.000000

YOFFSET 0.000000

YMIN -4.000000

YMAX 66.000000

YADJUST 1

YALTADJUST 1

END

Below are the values I am current running which seems to work fine. ( stock value is the blue , mine  are in the black )

ADDED PARAMETER TO THE PCMX  .RDT FILE TO ADJUST THE DECEL MAX PW FOR FUEL SHUTOFF. I raised this over the stock value.

BEGIN PARAM "P_048A Max pw Decel Fuel Shutoff nuetral"

ID "P_048A"

UNITS ""

ADDRESS 0x48A

TYPE 0x22

SCALE 262144.000000

OFFSET 0.000000

MATH 1

FORMAT "%4.5f"

ADJUST 1

ALTADJUST 1

END

BEGIN PARAM "P_048C Max pw Decel Fuel Shutoff drive"

ID "P_048C"

UNITS ""

ADDRESS 0x48C

TYPE 0x22

SCALE 262144.000000

OFFSET 0.000000

MATH 1

FORMAT "%1.5f"

ADJUST 1

ALTADJUST 1

END

Max pw decal for Fuel Shutoff  ( stock value is the blue , mine are in the black )

ADDED PARAMETER TO THE PCMX  .RDT FILE TO ADJUST  JUST STARTED DELAY TO START THE MAF CLIP.

BEGIN PARAM "P_57e1 Delay before maf clip"

ID "P_57e1"

UNITS ""

ADDRESS 0x57E1

TYPE 0x1

SCALE 1.000000

OFFSET 0.000000

FORMAT "%3.0f"

ADJUST 1

ALTADJUST 1

END

Start up Delay for Maf Clip  ( stock value is the blue , mine are in the black )