July 30, 2026

EEC Tuner tuning notes

Most of the information in this file is directed specifically at the A9L processor. However, the information is still applicable to the other processors in theory and description of operation.

This is a work in progress. Comments are welcomed and appreciated.

PIP

Think of this as a rev limiter using the following formula:

PIP = 6006250/maxRPM
maxRPM = 6006250/PIP

for example stock A9L

PIP	= 961 maxRPM = 6006250/961

= 6250 rpms

Lower numbers give a higher rev limit.

The PIP is a sample rate frequency that limits the PCM program operating loop to a minimum rate. If the loop is sampled less then PIP    times within one operating cycle (2 crankshaft revolutions), the PCM limits rpm.

With this logic, you should be able to set the pip to 1 and the car should run normally, but not limit RPM. I have tested this on an A9L processor.

WOT_VOLTAGE

The WOT_VOLTAGE defines the throttle position that WOT operating mode is entered. On A9L processors, the WOT_VOLTAGE is the measured TPS voltage minus the idle voltage. That makes the stock 2.71 volts really	~3.71 volts as read on the TPS signal line. TPS usually goes to 4.6V at full scale. So stock WOT operation is triggered at about 80% and up throttle. You can control when the WOT functions and scalars are used by adjusting this value.

The following tags are only active during WOT operating mode

WOT_SPARK_ADDEER

WOT_ADVANCE_VS_RPM

WOT_FUEL_MULTIPLIER_VS_RPM

WOT_ADVANCE_VS_ECT

WOT_ADVANCE_VS_ACT

CRANK_FUEL_PULSEWIDTH_MULTIPLIER_VS_TIME	    
( 8191.88, 1 )	( 0, 1 )	( 0, 1 )
( 0, 1 )	( 0, 1 )	( 0, 1 )

This function is used to modify the CRANK_FUEL_PULSEWIDTH_VS_ECT values. The x value is the time in seconds, the y is the multiplier. This allows across-the-ECT-range pulsewidth changes without having to enter new ECT-dependent pulsewidths.

This is useful for reducing the pulsewidth when using larger injectors. This dramatically improves engine starting, and the surge that comes right after starting with larger injectors. I suspect the surging is burning latent fuel from a wet manifold that was over-doused with fuel. Set the multiplier in proportion to the size of the new injector.

For example: If going from 19 to 36 lb/hr injectors. You might use the following function. 19/36 = c

CRANK_FUEL_PULSEWIDTH_MULTIPLIER_VS_TIME	    
( 8191.88, 0.53 )	( 0, 0.53 )	( 0, 1 )
( 0, 1 )	( 0, 1 )	( 0, 1 )

The smallest pulsewidth appears to be about 2ms. A lower multiplier will not lower the pulsewidth below 2ms. However, a zero multiplier produces a 0ms pulse.

Using the 0 multiplier you can introduce a period where the engine is cranking, but the fuel injectors are not introducing fuel -- just like WOT cranking without holding the throttle wide open. This can be useful for cars running very large injectors, such that a 2ms pulsewidth will still flood the engine on starting.

For example: I use this on my car with 50 lb/hr fuel injectors. Without this setting, it will not start when its warm -- I will have to start it at WOT to clear the flooding

CRANK_FUEL_PULSEWIDTH_MULTIPLIER_VS_TIME	    
( 8191.88, 0 )	( 3, 0 )	 ( 1.5, 0.35 )

( 1.5, 0 )	( 0.75, 0 ) ( 0, 0.35 )

This function would gradually, over 3/4 of a second, reduce the injector pulsewidth from 0.35 full value to 0. Then from 3/4 of a second to 1.5 seconds, no fuel would be delivered. From 1.5 seconds to 3 seconds fuel delivery would decrease from 0.35 full value to no fuel. From 3 seconds to 8191.88 (infinity in relation to cranking), no fuel would be delivered. This would clear any flooding condition as I crank beyond 3 seconds.

CRANK_FUEL_PULSEWIDTH_VS_ECT
( 65534, 0.00199893 )	( 180, 0.00199893 )	( 150, 0.0023003 )
( 70, 0.00569924 )	( 40, 0.010502 )	( 20, 0.0141489 )
( 0, 0.0218891 )	( -20, 0.0387617 )	( -65536, 0.0387617 )
Format: (ECT in degrees F, injector pulsewidth in seconds). Notice how dramatic the pulsewidth changes as the ECT changes. For example, on the A9L series a 10.5 ms pulsewidth at 40F changes to a 2.3ms pulsewidth at 150F. This may be a critical adjustment for larger-then-stock injectors and easy starting.

CLOSED_THROTTLE_SPARK_ADDER, PART_THROTTLE_SPARK_ADDER, WOT_SPARK_ADDER	-
GLOBAL_SPARK_ADDER
xxxSPARK_ADDERs can be positive to add timing, or negative to remove timing.

TIP_IN_RETARD A positive number pulls out that number of degrees right after a shift. A negative number here might, presumably, advance ignition after a shift. Probably a dangerous thing!

CHECKSUM_BASE_ADDRESS - This is the starting address used to calculate a checksum on the ROM. The checksum test processes blocks of code until it reaches a base address that is 0. Set this value to 0 to remove KOEO error code 15 (checksum test error). The ROM checksum is valid only on the stock calibration -- Once you change anything, the checksum is invalid and error code 15 is generated.

It is not known if the presence of error code 15 effects PCM operation -- such as not storing keep-alive memory.

OPEN_LOOP_IDLE_MULTIPLIER	1 #(ZA0 series)
This variable modifies the fuel mixture at idle when the vehicle is in OL mode 
(during cold start).    A lower value richens the mixture.
You might use this if you run a radical cam and the car won't idle smoothly until it gets warm

INTAKE_VOLUME                                                        5.5	 # Intake Manifold Volume
You should play with this a bit. It is a non-critical measurement.    People tell me it has an effect on throttle response. The GT40 has a larger volume -- try a value of 6. I don't know what the units are.

MAF_VOLTAGE                                                            4.78523	    # Max MAF voltage
MAF_VOLTAGE defines the maximum usable voltage as sent from the Mass Airflow Sensor signal line. This parameter essentially caps the maximum measurable airflow. When airflow reaches a rate such that the signal voltage is higher then MAF_VOLTAGE, the PCM fixes the fuel delivery rate as though MAF_VOLTAGE air is flowing. Raising this value from 4.7853 to 5.00 can increase the potential airflow measurement by 40%!. This assumes that your Mass airflow meter reports accurately at these higher rates.

FUEL_TABLE_ECT_SCALING	    #
( 254, 9 )	( 240, 9 )	( -30, 0 )
( -256, 0 )	( -256, 0 )	( -256, 0 )
This function defines the ECT values for the columns of the following tables:
      STARTUP_FUEL_TABLE
      BASE_FUEL_TABLE

FUEL_TABLE_LOAD_SCALING
 ( 1.99997, 7 )	( 0.899994, 7 )	( 0.700012, 5 )
( 0.399994, 3 )	( 0.149994, 1 )	( 0.0750122, 0 )
( 0, 0 )
This function defines the load (volumetric efficiency) values for the rows
of the BASE_FUEL_TABLE.

EMISSION_TABLE_LOAD_SCALING	    #
( 1.99997, 7 )	( 0.75, 7 )	( 0.600006, 6 )
( 0.100006, 1 )	( 0.0499878, 0 )	( 0, 0 )
This function defines the load (volumetric efficiency) values for the rows
of the following tables:

EXHAUST_PULSE_DELAY

HEGO_AMPLITUDE

HEGO_BIAS

EMISSION_TABLE_RPM_SCALING	    #
( 16383.8, 3 )	( 3000, 3 )	( 900, 1 )
( 700, 0 )	( 0, 0 )	( 0, 0 )
( 0, 0 )	( 0, 0 )
This function defines the RPM values for the columns of the following tables:

EXHAUST_PULSE_DELAY

HEGO_AMPLITUDE

HEGO_BIAS

SPARK_TABLE_RPM_SCALING	    #

( 16383.8, 9 )	( 4000, 9 )	( 3000, 8 )
( 1500, 5 )	( 500, 0 )	( 0, 0 )
( 0, 0 )
This function defines the RPM values for the columns of the following tables:

BASE_SPARK_TABLE

ALTITUDE_BASE_SPARK_TABLE

LIMP_MODE_SPARK_TABLE

INJECTOR_TIMING_TABLE

SPARK_TABLE_LOAD_SCALING	    #

( 1.99997, 7 )	( 0.75, 7 )	( 0.600006, 6 )

( 0.100006, 1 )	( 0.0499878, 0 )	( 0, 0 )

( 0, 0 )

This function defines the load (volumetric efficiency) values for the rows

of the following tables:

BASE_SPARK_TABLE

ALTITUDE_BASE_SPARK_TABLE

LIMP_MODE_SPARK_TABLE

INJECTOR_TIMING_TABLE

NEUTRAL_IDLE                                                          672	    # Neutral Idle RPM

DRIVE_IDLE                                                              672	    # Drive Idle RPM

Defines the target base idle when the car is in Neutral and Drive. DRIVE_IDLE is used for automatic transmission equipped vehicles -- it has no effect on cars with manual transmissions.

ENGINE_DISPLACEMENT                                            301.082	    # Engine Displacement
Defines the cubic inch displacement of the engine. This parameter us important because is defines how load is calculated. The VE(Load) is calculated by the airflow, the RPM and the ENGINE_DISPLACEMENT. Engine_displacement/(Airflow/RPM) = VE(load)

Increasing ENGINE_DISPLACEMENT typically causes increased ignition advance and leaner mixtures. This parameter is closely coupled with the MAF_FUNCTION. Double this value, and you halve the calculated load value -- that means leaner fuel mixtures and more timing. Be careful here!

LOW_INJECTOR_SLOPE                                26.0995	    # Injector slope lb./hr

This parameters compensates for the physical limitations of a particular fuel injector for unusually small pulsewidths (<~4ms). In general, real life fuel injectors are sloppy when the commanded pulsewidth is close to the duration of the opening and closing event time for the injector solenoid. Typical injectors take about 1 ms to open and 1 ms to close. That is a total open/close event duration of 2 ms. When the command pulsewidth is in this low-pulsewidth range (<~4 ms), the real-life fuel delivery is not linear to the rated full flow injector delivery rate. The LOW_INJECTOR_SLOPE is a fudge-factor that accounts for this deviation.

Injector pulsewidths are lowest when the RPM is high, and the throttle is closed. Idle pulsewidths are in the 2-4ms range. Larger injectors decrease all pulsewidths in direct proportion to the size increase. Double the injector size, halve the pulsewidths for any given airflow.

HIGH_INJECTOR_SLOPE(ZA0-series)INJECTOR_SIZE(all others

The fully-open fuel delivery rate in pounds(mass)/Hour, at 45 psi, of the fuel injector.

INJ_DELAY		0	    # Global injector delay (crank deg.)

GLOBAL_ACCEL_MULTIPLIER		0.398438	    # Global Accel pump multiplier

FAN_TEMP		220	    # Coolant temp for Electric Fan (Deg F)

Fan operation temperature in degrees Fahrenheit.

FAN_ENABLE		0	    # Electric Fan control enable(0=no, 1=yes)
1 enables fan operation, 0 disables

LOW_DWELL		0.500008	    # Max low speed dwell (off time in ms)
Coil charge time in milliseconds. Don't know what the purpose of this is.

HIGH_DWELL		0.2	    # Max high speed dwell (off time in ms)
Coil charge time in milliseconds. Don't know what the purpose of this is.

OPEN_LOOP_FUEL_MULTIPLER          0.996094	    # Global open loop fuel multiplier
Across-the-board fuel mixture adjustment when operating in Open Loop mode (non HEGO controlled operation). Numbers less then one richen the mixture (increase fuel delivery) Numbers greater then one lean the mixture (decrease fuel delivery)

EGR_MULTIPLIER		1	    # EGR multiplier

This is a gross scaling adjustment for the flow rate of a given EGR valve. Use a number larger then one to proportionately add more recirculated exhaust gas to the intake charge.    Numbers less then one reduce the EGR flow. 

For example, setting EGR_MULTIPLIER to 0.75 would deliver three fourths the amount of exhaust compared to stock. A setting of 1.5 would deliver 1 and a half times the EGR flow. Don't know why you would use this, but its fun to change the values and log EVP sensor output (pin 27).

EGR_TYPE		0	    # EGR type (0=sonic, 1-PFE, 2=none)
Set to 2(none) to remove the EGR related error codes for a removed or non-functioning EGR. The effect on fuel mixture and ignition timing is unknown -- probably stays exactly the same.

The Purge canister solenoid still works when the Thermactor and EGR are set to none. The Purge Canister solenoid opens at off throttle, car warm, and running for 5 minutes or so.

SECOND_SPEED_LIMITER_ON

The _ON must be set higher then the _OFF for this function to work. The engine cuts out when the speed on the speedometer reaches the speed set with SECOND_SPEED_LIMITER_ON, the engines starts running after the speed falls off to SECOND_SPEED_LIMITER_OFF

FIRST_SPEED_LIMITER_### 
Doesn't seem to do anything.
Possibly used on automatic equipped mustangs, or when the car is under/over operating temperature.

HALF_FUEL_REV_LIMIT_ON		8000	    # Turn on half-fuel rev limit
HALF_FUEL_REV_LIMIT_OFF		7500	    # Turn off half-fuel rev limit
The _ON must be set higher then the _OFF for this function to work. The PCM cuts fuel delivery in half when the engine speed reaches the RPM    of the _ON value. Fuel delivery remains cut in half until the engine speed drops below the RPM set in the _OFF speed.

NO_FUEL_RPM		7000	    # Max RPM (no fuel)
Cuts all fuel when this RPM is reached. Harsh rev limiter. Avoid its use.

MIN_LOAD_FOR_CLOSED_LOOP	0.00500488	    # Minimum load for CL control
?

TIME_AT_HIGH_LOAD_BEFORE_OPEN_LOOP	2 # Time at high load to force OL (sec)

The TIME_AT_HIGH_LOAD_BEFORE_OPEN_LOOP      is the numbers of seconds that the car will stay in closed loop (HEGO fuel control) when it detects a high load demand (anything over about 1/3 throttle). A 0 works here also.

THERMACTOR_PRESENT	1	    # Thermactor present (0-no, 1-yes)
Set to 0 to remove the Thermactor related error codes for a removed air pump belt or non-functioning air-diverter/air-bypass valves or their associated solenoids.

NUMBER_OF_HEGOS	2	    # Number of HEGO's
Set to zero to force Open Loop fuel delivery operation?

MIN_ECT_FOR_FUEL_SHUTOFF	140	# Min ECT for decel fuel shutoff (Deg F)

MIN_RPM_FOR_FUEL_SHUTOFF	150	# Min RPM for decel fuel shutoff (Deg F)

MIN_ACT_FOR_ADAPTIVE_CONTROL	100	# Min ACT for Adaptive Control (Deg F)

MAX_ACT_FOR_ADAPTIVE_CONTROL	200	# Max ACT for Adaptive Control (Deg F)

?Does Anyone use these? The Cobra (X3Z) changes these values from the A9L, but I don't know why?

MAF_FUNCTION	# Mass Air Transfer Function
( 15.9998, 835.509 )	( 4.76807, 835.509 )	( 4.44312, 695.465 )
( 4.177, 595.977 )	( 3.88599, 499.658 )	( 3.54395, 399.219 )
( 3.35498, 347.574 )	( 3.14893, 295.612 )	( 2.90991, 243.334 )
( 2.69409, 201.828 )	( 2.58203, 182.5 )	( 2.44995, 161.272 )
( 2.31592, 142.261 )	( 2.15405, 120.083 )	( 1.98901, 100.122 )
( 1.88989, 90.2996 )	( 1.80298, 82.3786 )	( 1.69092, 72.8734 )
( 1.57397, 62.4176 )	( 1.46802, 54.1798 )	( 1.32104, 44.6745 )
( 1.18188, 36.7535 )	( 1.09106, 32.6346 )	( 0.884033, 23.7631 )
( 0.75, 18.6936 )	( 0.571045, 13.6242 )	( 0, 13.6242 )
( 0, 13.6242 )	( 0, 13.6242 )	( 0, 13.6242 )


The x value is the Voltage on the MAF sig line. The y coordinate is the Kg/Hr flow rate. This allows you to use the data provided by Pro M (Best Products Inc.)

The following functions can be used to estimate flow data for various injector calibrations. V is the voltage read between the MAF sig (pin 50) and MAF return (pin 9). ** means "raised to the power of".

| Calibration | Flow equation (Kg/hr) |
| --- | --- |
| 19 lb/hr | Flow = 1.4925(1.384+V)**3.5. |
| 24 lb/hr | Flow = 1.990(1.384+V)**3.5. |
| 36 lb/hr | Flow = 2.85(1.384+V)**3.5. |

WOT_ADVANCE_VS_RPM	    # WOT spark Advance vs. RPM
( 16383.8, 26 )	( 5000, 26 )	( 3500, 22 )
( 2600, 22.5 )	( 2150, 21.5 )	( 1800, 18.5 )
( 1000, 8 )	( 0, 8 )
The x coordinate is the RPM. The y coordinate is the ignition advance. This is the same advance that you would read off the crankshaft timing marks with a timing light -- the initial timing is included.

ACCELERATOR_PUMP_VS_TP_VOLTAGE	# Accelerator Enrichment Multiplier vs. TP voltage
( 4.98047, 0 )	( 4.16016, 0 )	( 2.20703, 0.296875 )
( 0.976562, 0.953125 )	( 0, 1 )
The x coordinate is the TP Voltage, the y coordinate is the enrichment multiplier.

WOT_FUEL_MULTIPLIER_VS_RPM	# WOT Fuel Multiplier vs. RPM
( 16383.8, 0.945312 )	( 6000, 0.945312 )	( 4400, 0.953125 )
( 3800, 0.976562 )	( 3200, 0.945312 )	( 2400, 0.96875 )
( 1600, 0.90625 )	( 0, 1 )

The WOT_FUEL_MULTIPLIER_VS_RPM function richens the mixture with numbers less then one. Numbers greater then on lean the mixture.

ACCELERATOR_PUMP_FUEL_TABLE	# Accelerator Enrichment Fuel Table(LB/min)
			Throttle Rate (Deg/Sec)

| ECT | 16 | 32 | 48 | 64 | 80 | 96 | 112 | 128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -30 | 21 | 23 | 26 | 28 | 31 | 34 | 39 | 56 |
| 0 | 12 | 14 | 16 | 18 | 26 | 30 | 38 | 55 |
| 30 | 5 | 8 | 10 | 12 | 16.5 | 21 | 27 | 45 |
| 60 | 4 | 5 | 6 | 8 | 12.5 | 17 | 25.5 | 34 |
| 90 | 2 | 2 | 3 | 4.5 | 8 | 12.5 | 18 | 29 |
| 120 | 1.5 | 1.5 | 2 | 3 | 4.5 | 8 | 12 | 20 |
| 150 | 1.5 | 1.5 | 2 | 3 | 4.5 | 8 | 12 | 18 |

STARTUP_FUEL_TABLE	# Startup Fuel Table (A/F ratio)
	ECT

| Seconds | -30 | 0 | 30 | 60 | 90 | 120 | 150 | 180 | 210 | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3.375 | 4.125 | 3 | 1.875 | 1.875 | 1.875 | 1.875 | 1.375 | 1.875 | 3.375 |
| 4 | 2.875 | 3.625 | 2.625 | 1.875 | 1.875 | 1.875 | 1.875 | 1.625 | 1.875 | 2.875 |
| 8 | 2.75 | 2.75 | 2.375 | 1.875 | 1.875 | 1.875 | 1.875 | 1.5 | 1.875 | 2.75 |
| 12 | 2.25 | 2.125 | 2.125 | 1.875 | 1.875 | 1.875 | 1.875 | 1.25 | 1.875 | 2.25 |
| 16 | 1.625 | 1.625 | 1.5 | 1.375 | 1.25 | 1.375 | 1.375 | 1.125 | 1.375 | 1.625 |
| 20 | 1.25 | 1.5 | 1.125 | 1 | 1 | 1.25 | 1.25 | 0.875 | 1 | 1.25 |
| 24 | 1 | 1.25 | 0.875 | 0.5 | 0.375 | 0.875 | 0.875 | 0.25 | 0.5 | 1 |
| 28 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The STARTUP_FUEL_TABLE uses larger numbers to richen the mixture.

BASE_FUEL_TABLE	# Base Fuel Table (A/F ratio)(Load vs ECT)
	ECT

| Load (VE) | -30 |  | 0 |  | 30 |  | 60 |  | 90 |  | 120 |  | 150 |  |  | 180 | 210 |  | 240 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.08 (8%) |  | 17.875 |  | 17.875 |  | 17.675 |  | 17.25 |  | 16.625 |  | 16.375 |  | 16 | 15.25 |  |  | 15.25 |  | 15.25 |
| 0.15 (15%) |  | 17.625 |  | 17.625 |  | 17 |  | 16.625 |  | 16.5 |  | 16.375 |  | 16 | 15.25 |  |  | 15.25 |  | 15.25 |
| 0.27 (27%) |  | 14.125 |  | 14.25 |  | 14.25 |  | 14.375 |  | 14.5 |  | 14.75 |  | 14.75 | 15.25 |  |  | 15.25 |  | 14.375 |
| 0.40 (40%) |  | 13.625 |  | 14.375 |  | 14.375 |  | 14.375 |  | 14.875 |  | 15.25 |  | 15.25 | 16 |  |  | 16 |  | 14.75 |
| 0.55 (55%) |  | 12.75 |  | 13.875 |  | 14.25 |  | 14.375 |  | 14.875 |  | 14.875 |  | 15.25 | 16 |  |  | 16 |  | 15 |
| 0.70 (70% |  | 12.125 |  | 12.125 |  | 13.5 |  | 13.5 |  | 13.875 |  | 14.375 |  | 14.125 | 14.25 |  |  | 13.625 |  | 13.625 |
| 0.80 (80%) |  | 11.25 |  | 11.25 |  | 12.75 |  | 12.75 |  | 13.125 |  | 13.5 |  | 13.75 | 13.75 |  |  | 13.625 |  | 13.5 |
| 0.90 (90%) |  | 10.5 |  | 10.5 |  | 12.5 |  | 12.5 |  | 12.75 |  | 13.125 |  | 13.5 | 13.5 |  |  | 13.5 |  | 13.5 |

BASE_SPARK_TABLE		# Base Spark Table (Deg BTDC)(Load vs RPM)

|  |  |  |  | RPM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load (VE) | 500 | 700 | 900 | 1100 | 1300 | 1500 | 2000 | 2500 | 3000 | 4000 |
| 0.05 (5%) | 28 | 28 | 28 | 31 | 33 | 34 | 34 | 34 | 34 | 34 |
| 0.10 (10% | 28 | 28 | 28 | 31 | 33 | 34 | 34 | 34 | 28 | 28 |
| 0.20 (20%) | 28 | 28 | 33 | 36 | 36 | 37 | 38 | 38 | 28 | 28 |
| 0.30 (30%) | 25 | 26 | 28 | 29 | 32 | 32 | 38 | 40 | 28 | 28 |
| 0.40 (40%) | 20 | 21 | 25 | 28 | 31 | 32 | 33 | 34 | 28 | 28 |
| 0.50 (50%) | 15 | 15 | 16 | 16 | 18 | 20 | 23 | 27 | 27 | 27 |
| 0.60 (60%) | 8 | 10 | 10 | 11 | 13 | 15 | 21 | 21 | 21 | 21 |
| 0.75 (75%) | 8 | 8 | 9 | 10 | 12 | 14 | 18 | 21 | 21 | 21 |

ALTITUDE_BASE_SPARK_TABLE	    # Altitude base spark table.

|  |  |  |  | RPM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load (VE) | 500 | 700 | 900 | 1100 | 1300 | 1500 | 2000 | 2500 | 3000 | 4000 |
| 0.05 (5%) | 28 | 28 | 28 | 31 | 33 | 34 | 34 | 34 | 34 | 34 |
| 0.10 (10% | 28 | 28 | 28 | 32 | 34 | 35 | 35 | 34 | 28 | 28 |
| 0.20 (20%) | 28 | 28 | 33 | 37 | 37 | 38 | 38 | 38 | 28 | 28 |
| 0.30 (30%) | 25 | 26 | 30 | 37 | 38 | 36 | 36 | 38 | 28 | 28 |
| 0.40 (40%) | 20 | 21 | 25 | 32 | 34 | 34 | 33 | 32 | 28 | 28 |
| 0.50 (50%) | 15 | 16 | 16 | 16 | 18 | 20 | 26 | 27 | 27 | 27 |
| 0.60 (60%) | 8 | 8 | 9 | 10 | 11 | 14 | 21 | 21 | 21 | 21 |
| 0.75 (75%) | 8 | 8 | 9 | 10 | 11 | 13 | 21 | 21 | 21 | 21 |

LIMP_MODE_SPARK_TABLE	    # Limp Mode Spark Table (Deg BTDC)(Load vs RPM)

|  |  |  |  | RPM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load (VE) | 500 | 700 | 900 | 1100 | 1300 | 1500 | 2000 | 2500 | 3000 | 4000 |
| 0.05 (5%) | 28 | 28 | 28 | 31 | 33 | 34 | 34 | 34 | 34 | 34 |
| 0.10 (10% | 28 | 28 | 28 | 31 | 33 | 34 | 34 | 34 | 34 | 34 |
| 0.20 (20%) | 25 | 26 | 30.5 | 33.5 | 35 | 36.5 | 38 | 38 | 38 | 38 |
| 0.30 (30%) | 20 | 21 | 20 | 24 | 31 | 32 | 33 | 33 | 33 | 33 |
| 0.40 (40%) | 14 | 14 | 14 | 14 | 16 | 21 | 27 | 27 | 27 | 27 |
| 0.50 (50%) | 8 | 9 | 10 | 11 | 13 | 17 | 22 | 22 | 22 | 22 |
| 0.60 (60%) | 6.5 | 8 | 9 | 10 | 12 | 14 | 18 | 18 | 18 | 18 |
| 0.75 (75%) | 6.5 | 6.5 | 9 | 10 | 12 | 14 | 18 | 18 | 18 | 18 |

INJECTOR_OUTPUT_PORT	# Injector Output Port Table

	Cylinder

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 |

WOT_ADVANCE_VS_ECT	    #
( 254, -4 )	( 246, -4 )	( 236, -2 )
( 200, 0 )	( 120, 0 )	( 86, 3 )
( -256, 3 )
The x coordinate is the ECT, the y coordinate is the timing in degrees that is added to the current timing to account for changes in engine coolant temperature. This function is only used at WOT -- defined by the WOT_VOLTAGE.

WOT_ADVANCE_VS_ACT	    # 
( 254, -6 )	( 240, -6 )	( 150, 0 )
( -256, 0 )	( -256, 0 )	( -256, 0 )
This function is only active during WOT operation node. The x coordinate is the ECT in farenheit, the y-coordinate is the amount of ignition advance that is added (or subtracted if the number is negative) to the overall ignition advance.

OL_FUEL_MULTIPLIER_VS_ACT	    # 
( 254, 1 )	( 76, 1 )	( 0, 0.8125 )
( -40, 0.640625 )	( -256, 0.640625 )	( -256, 0.640625 )
This function is only active during OL operation node. Open Loop operation occurs at throttle postions of greater then one third throttle. The x coordinate is the ECT in farenheit, the y-coordinate is fuel multiplier. Multipliers lower then one richen the mixture(more fuel), numbers greater then one lean the mixture (less fuel).

PT_ADVANCE_VS_ACT	    #
( 254, -2 )	( 190, -2 )	( 160, 0 )
( -256, 0 )	( -256, 0 )
This function is only active during Part Throttle (not WOT) operation node. The x coordinate is the ECT in farenheit, the y-coordinate is the amount of ignition advance that is added (or subtracted if the number is negative) to the overall ignition advance.

ADVANCE_VS_BP	    #
( 31.875, 0 )	( 27.75, 0 )	( 26.5, 4 )
( 23.5, 4 )	( 0, 12 )
The x coordinate is the barometric pressure in inches of mercury, the y-coordinate is the amount of ignition advance that is added (or subtracted if the number is negative) to the overall ignition advance.

ADVANCE_RATE_VS_RPM	    #
 ( 8160, 1 )	( 4000, 1 )	( 3008, 1.5 )
( 992, 2 )	( 0, 2 )	( 0, 2 )
The predicted advance rate between table lookups? Higher numbers here give a noticable increase in throttle response. Especially on modified cars with camshafts and high stall converters.

MIN_LOW_SPEED_DWELL	    # 
( 4095.94, 0.00279999 )	( 15, 0.00279999 )	( 14, 0.00300027 )
( 12, 0.00399972 )	( 10, 0.00449944 )	( 0, 0.00449944 )

MIN_HIGH_SPEED_DWELL	    #
( 4095.94, 0.00300027 )	( 15, 0.00300027 )	( 13, 0.00334931 )
( 12, 0.00370026 )	( 10, 0.00499917 )	( 0, 0.00499917 )

DASHPOT_CLIP	    # 
( 16383.8, 1.6001 )	( 2000, 1.6001 )	( 1500, 1 )
( 500, 0.120117 )	( 0, 0.120117 )

SEA_LEVEL_LUGGING_OL_FUEL_MULTIPLIER	    # 
( 16383.8, 1 )	( 1500, 1 )	( 1200, 1.20312 )
( 750, 1.20312 )	( 650, 1 )	( 0, 1 )

ALTITUDE_LUGGING_OL_FUEL_MULTIPLIER	    # 
( 16383.8, 1 )	( 0, 1 )	( 0, 1 )
( 0, 1 )	( 0, 1 )

INJECTOR_OFFSET_VS_BATTERY_VOLTAGE	    # 
( 15.9375, 0.90625 )	( 14, 0.90625 )	( 13, 1 )
( 12, 1.15625 )	( 11, 1.40625 )	( 10, 1.6875 )
( 9, 2.03125 )	( 8, 2.59375 )	( 7, 3.53125 )
( 6, 5.53125 )	( 5.5, 7.5 )	( 0, 7.5 )

OL_FUEL_MULTIPLIER_VS_RPM	    #
( 16383.8, 0 )	( 1100, 0 )	( 1075, 1 )
( 0, 1 )	( 0, 1 )	( 0, 1 )

DASHPOT_DECREMENT_RATE	    # 
( 15.9998, 0.0625 )	( 0.75, 0.0625 )	( 0.649902, 0.0449219 )
( 0.449951, 0.00708008)	( 0.350098, 0.00195312 )	( 0.0620117, 0.00292969 )
( 0, 0.00341797 )

INJECTOR_TIMING_TABLE	# Injector timing table Crank degrees
	RPM

| Load (VE) | 500 | 700 | 900 | 1100 | 1300 | 1500 | 2000 | 2500 | 3000 | 4000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.05 (5%) | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 |
| 0.10 (10% | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 |
| 0.20 (20%) | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 |
| 0.30 (30%) | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 | 352 |
| 0.40 (40%) | 352 | 352 | 352 | 352 | 352 | 380 | 400 | 400 | 400 | 400 |
| 0.50 (50%) | 380 | 380 | 380 | 380 | 380 | 380 | 420 | 420 | 464 | 464 |
| 0.60 (60%) | 420 | 420 | 420 | 420 | 420 | 420 | 420 | 464 | 464 | 464 |
| 0.75 (75%) | 420 | 420 | 420 | 420 | 420 | 420 | 420 | 464 | 464 | 464 |

INJECTOR_FIRING_ORDER	    # Injector firing order

1	3	7	2	6	5	4	8

EXHAUST_PULSE_DELAY	 # in Engine Revolutions
This is the amount of time in engine revolutions that it takes the exhaust charge to reach the oxygen sensor after ignition. Increase this value to improve idle on cars with long tube headers.

Here is a more specific definition contributed from a someone in the EEC-EFI mailing list: "EXHAUST_PULSE_DELAY is the time between when the injector
pulse width is calculated in the EEC (*not* when the injector
is fired), to when that pulse ends up reliably sensed at the HEGO
sensor. This is used to predict when the O2 signal will cross so
it can get the correct amplitude and bias results."

From an EEC Tuner user point of view, longer headers mean larger values in the table. Conceivably, changing the injector timing table could also have any effect here (I would presume only a negligible one though < 1).

|  |  | RPM |  |  |
| --- | --- | --- | --- | --- |
| Load (VE) | 700 | 900 | 1950 | 3000 |
| 0.05 (5%) | 10 | 10 | 10 | 10 |
| 0.10 (10% | 10 | 10 | 10 | 10 |
| 0.20 (20%) | 10 | 10 | 10 | 10 |
| 0.30 (30%) | 10 | 10 | 10 | 10 |
| 0.40 (40%) | 10 | 10 | 10 | 10 |
| 0.50 (50%) | 10 | 10 | 10 | 10 |
| 0.60 (60%) | 10 | 10 | 10 | 10 |
| 0.75 (75%) | 10 | 10 | 10 | 10 |

HEGO_AMPLITUDE
Description taken from an EEC-EFI mailing list contributor:
" HEGO_AMPLITUDE is the desired amplitude of the O2 dither signal. This is modified primarily for emissions - it is set up to optimize the catalytic converter performance. A certain minimum is required to maintain the system in a limit cycle oscillation."

|  |  | RPM |  |  |
| --- | --- | --- | --- | --- |
| Load (VE) | 700 | 900 | 1950 | 3000 |
| 0.05 (5%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.10 (10% | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.20 (20%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.30 (30%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.40 (40%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.50 (50%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.60 (60%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |
| 0.75 (75%) | 0.0332031 | 0.0332031 | 0.0332031 | 0.0332031 |

HEGO_BIAS

Description taken from an EEC-EFI mailing list contributor:
"The HEGO_BIAS allows the averaged air-fuel ratio to be shifted slightly rich or slightly lean. This makes virtually no difference in power but it has a huge impact on emissions. This is used to adjust the CO-HC vs NOx balance on the emissions test cycle. A rich mixture will lower NOx but raise CO, a lean mixture will do the opposite."

|  |  | RPM |  |  |
| --- | --- | --- | --- | --- |
| Load (VE) | 700 | 900 | 1950 | 3000 |
| 0.05 (5%) | 0 | 0.00683594 | 0.00683594 | 0.00683594 |
| 0.10 (10% | 0 | 0.00195312 | 0.00195312 | 0.00195312 |
| 0.20 (20%) | 0 | 0 | 0 | 0 |
| 0.30 (30%) | 0 | 0.123047 | 0.123047 | 0.123047 |
| 0.40 (40%) | 0 | 0.123047 | 0.123047 | 0.123047 |
| 0.50 (50%) | 0.123047 | 0.121094 | 0.121094 | 0.121094 |
| 0.60 (60%) | 0.123047 | 0.119141 | 0.119141 | 0.119141 |
| 0.75 (75%) | 0.117188 | 0.117188 | 0.117188 | 0.117188 |

END