|  | 2.3 Turbo EEC-4 Specifications, Part Numbers, Calibration Codes, and Pinouts Restoration Progress and Photo Archive The Official 20th Anniversary Mustang Website & Registry 85~89 Merkur; 83~86 T-Bird 83~84.5 Mustang GT/GT350; 84~85 SVO Mustang; 85.5~86 SVO Mustang; 87~88 T-Bird; Application; Name; Misc Info. 1; 1; 1; 1; 1; Keep Alive Power; KAPWR. -; -; -; -; 2; Brake On-Off; BOO. 4; 4; 4; 4; 4; Ignition Diagnostic Monitor; IDM. -; -; -; -; 6; Cruise Control; VSS-DIF (-). 7; 7; 7; 7; 7; Engine Coolant Temerature; ECT. 10; 10; 10; 10; 10; A/C Clutch; ACC. 16; 16; 16; 16; 16; Ignition Ground; IGN GND. 17; 17; 17; 17; 17; Self Test Output; STO. 20; 20; 20; 20; 20; Case Ground; CSE GND. 21; 21; 21; 21; 21; Idle Speed Control; ISC. 22; 22; 22; 22; 22; Fuel Pump Control; FP. 23; 23; 23; 23; 23; Knock Sensor; KS. 25; 25; 25; 25; 43; Vane Air Temerature; VAT. 26; 26; 26; 26; 26; Voltage Reference; Vref. 29; 29; 29; 29; 29; Exhaust Gas Oxygen Sensor; EGO. 30; 30; -; -; 30; Neutral Drive Switch (auto) Neutral Input (stick); NDS NI. 32; 32; 32; 32; 31; Boost Control; BOOST; Some Early 1984 SVO's use a seperate box to operate the BCS. 34; -; -; -; 34; Data Output Link; DOL; used for trip/mpg accesory. 35; 35; 35; 35; 33; EGR Shutoff; EGR S/O. 36; 36; 36; 36; 36; Spark Output; SPOUT. 37; 37; 37; 37; 37; Vehicle Power; V PWR. 40; 40; 40; 40; 40; Battery Ground; BATT GND. -; -; -; -; 42; Kick Down Solinoid; KDS; Automatic Tranny only. 43; 43; 43; 43; 27; Vane Air Flow; VAF. 45; 45; 45; 45; 45; Barometric Pressure; BP; aka. BAP sensor. 46; 46; 46; 46; 46; Signal Return; SIG RET. 47; 47; 47; 47; 47; Throttle Position; TP; aka TPS (throt. pos. sensor). 48; 48; 48; 48; 48; Self Test Input; STI. 49; 49; 49; 49; 49; EGO Ground; EGO GND. -; -; -; -; 52; high-electro drive fan; HEDF. -; -; -; -; 53; locking upshift solinoid; LUS. 54; 54; 54; -; 54; WOT A/C Cutoff; WAC. -; -; -; -; 55; Eletro Drive Fan; EDF. 58; 58; 58; 58; 58; Injector Bank 1; INJ 1. 59; 59; 59; 59; 59; Injector Bank 2; INJ 2. 60; 60; 60; 60; 60; Battery Ground; BATT GND. -; -; 30; 30; 24; Octane Switch; OCT ADJ. -; -; -; -; 25; Air Charge Temp; ACT; Measured after the Intercooler. -; -; -; -; 35; Programmed Ride Control; ACL. |
| --- | --- |